# Rithik Subhash & 20486047

# MMME3085 – Initial Project Planning: Drawing Robot Software.

# Outline of the Problem to be Solved

The program will read a single-stroke font from `SingleStroke.txt’, which will be stored in memory, and then convert the input text into G-code that the drawing robot (via an Arduino) can execute. Each character in the font is defined by a sequence of relative moves (X, Y) with a pen state (0=up or 1 = down). At the run time, the user will supply a letter height in millimetres. The allowed range for that height is 4-10mm. A scale factor `height/18` is then applied to all font coordinates because the font’s nominal carp height is 18 units.

The program will read the message that will be written from a text file of arbitrary length. It will then process the file incrementally, so very long inputs are supported. The horizontal placement uses the final offset provided by each character’s data as in the last move set the start of the new character. Words will have to be laid out so they fit within a 100mm width drawing area without breaking words across the edge; if a new word exceeded 100mm, a new line is started before that word. Vertical layout uses a fixed line spacing of 5mm. The Carriage return (CR/ASCII 13) resets the X origin to the left margin; the line feed (LF/ASCII 10) moves the baseline down by 5mm. When dealing with the Y coordinate system, as the robot moves down the page, it makes Y more negative; going up the page makes Y more positive.

The G-code generation follows a small subset: The feedrate (F1000) and enable(M3) are sent once at the start. S0 raises the pen, S1000 lowers the pen. G0 X Y makes rapid non-drawing moves. G1 X Y draws lines. Pen commands are stateful and only sent when the state changes. Each G-code line is transmitted over a virtual serial port on Windows; the program essentially waits for an “okay” acknowledgement before sending the next line to respect the device buffer.

On completion, the program ensures the pen is up and returns the arm to (0,0). This submission is a design-only plan (no code): data structures, function prototypes, tests and flowcharts to guide the later implementation.

# Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| Stroke | struct {int16_t dx; int16_dy; uint8_t pen;} | Encodes one relative move from the font (change in x, change in y) plus pen state (0/1). Fixed-width ints match small coordinate ranges and avoid waste. One step in drawing a letter: how far to move in X and Y, and whether the pen is up or down. |
| Glyph | struct { uint8_t ascii; uint16_t nStrokes; Stroke* strokes; int16_t advanceUnits; } | Holds all strokes for one ASCII character; dynamic array because stroke count varies. advanceUnits caches the character’s horizontal advance (usually last X) for quick width estimates. Everything needed to draw one letter, plus how much space it takes before the next letter starts. |
| glyphLib | Glyph[128] | Direct ASCII-indexed lookup table for all glyphs (0(1) access during generation). A ready to use shelf of every letter so we can grab any letter`s recipe fast. |
| textHeightMM | int | User input(4-10mm). Integer keeps validation simple and matches spec. The size of the letters in millimetres that the user chooses. |
| scaletoMM | double | textHeightMM/18.0 converts font units to millimetres with sub-mm precision. The “zoom level” that turns font numbers into real sizes on paper. |
| cursorXmm, cursorYmm | double | The absolute drawing origin for the next stroke/glyph; required to convert relative font moves into absolute G-code coordinates. Tells where the pen is right now on the page (left-right and up-down in mm). |
| penIsDown | uint8_t | Tracks the pen state to suppress redundant S0/S1000 emissions. Shows whether the pen is currently touching the paper. |
| LINE_SPACING_MM | const double | Fixed inter-line offset to 5mm per spec; avoids magic numbers and centralises a constraint. The gap between lines of text. |
| MAX_DRAW_WIDTH_MM | const double | 100 is the hard limit for line wrapping. Used to decide wrap before drawing. The maximum width allowed for a line; if a word would exceed it, a new line would be started. |
| ASCII_CR, ASCII_LF | const int | The explicit control codes to reset X 13 (CR) and step Y by line spacing 10 (LF). Special new line signals in the text file. |
| gcLine | char[100] | Reusable formatting buffer for ASCII G-code lines before sending. A small text box where we build each command to the robot. |
| COM_INDEX | int | RS-232 library handle/index for the Windows virtual COM port (open/write/read). Which USB/COM port the robot is plugged into. |
| wordWidthMM | double | Temporary accumulator that sums advanceUnits * scaleToMM for the next word to decide wrap. A quick calculator to check if the next word will still fit on this line. |
| fontPath,textPath | char[] | File locations for the font and input text; enables testing with different sources. Where the font file and the message file live on your computer. |

# Function Declarations

- int loadGylphLibrary(const char * fontPath, Glyph glyohLib[], size_t libsize)

Parameters:

- fontPath – Input: path to SingleStrokeFont.txt / where the font file is

- glyphLib- Output/Changed: array filled with all glyph stroke data/ the array to fill with all letters

- libSize – Input: size of glyphLib (expecting 128)/ showing how many slots that the array has (128)

Return value: 1 if the font file is parsed and loaded correctly; 0 if the file can’t be opened or its format is invalid.

- Double computeScaleMM(int textHeightMM)

Parameters:

- textHeightMM – Input: desired letter height in mm (4-10).

Return value: Scale factor in mm per font unit. Returns 0.0 if input is out of range (error to be handled by caller)/ the zoom factor in mm per font unit (textHeightMM/18.0); 0.0 if the height is invalid.

- double estimateWordWidthMM ( const char* word, const Glyph glyphLib[], doublescaletoMM)

Parameters:

- word – input: C- string for the next word (no spaces)/the next word

- glyphLib – Input: loaded glyph library / the letter library

- scaletoMM – Input: scale from computeScaleMM / the zoom factor

Return value: Estimated width in mm obtained by summing each glyph’s advancedUnits * scaleToMM. Returns -1.0 if a glyph is missing/ how wide that word will be on paper(mm).

- int drawGlyph(char c, const Glyph glyphLib[], double scaletoMM, double* cursorXmm, double*cursorYmm, uint8_t* penIsDown)

Parameters:

- C – Input: the character to draw.

- glyphLib – Input: loaded strokes for all glyphs / the letter library

- scaleToMM – Input: scale factor / zoom factor

- cursorXmm, cursorYmm – Changed: current absolute position; cursorXmm advances to the glyph’s end (next origin)/ where the pen is now (X/Y) and whether it’s up or down

- penIsDown – Changed: toggled to match stroke pen states (0/1); X advances to the end of this letter.

Return value: 1 on success; 0 if the glyph doesn’t exist or send fails. / 1 if the commends were sent and 0 if the letter is missing or sending failed.

- void applyNewLine(double* cursorXmm, double* cursorYmm, double lineSpacingMM)

Parameters:

- cursorXmm – Changed: set to 0.0

- cursorYmm – Changed: decremented by lineSpacingMM (5.0)

- lineSpacingMM – Input: Vertical step between lines.

Return value: None.

- int beginGcodeSession(void)

Parameters: none

Return value: 1 after successfully sending startup sequence (F100,M3,S0) and receiving “ok” for each; 0 on failure/ 1 if the robot accepted the startup commands(F1000,M3,S0), 0 otherwise.

- int emitGcode(const char* fmt,…)

Parameters:

- fmt – input:printf-style format for single G-code line

- … - input: values for the format. Populates gcline and sends it.

Summary of input: One formatted G-code to send (e.g., G1 X… Y…)

Return value: 1 if the line is sent and an “ok” is received; 0 on timeout/serial error. Essentially this is a thin wrapper so the main logic stays clean and the code works in serial.

- int processTextFile(const char* textPath, const Glyph glyphLib[], double scaleToMM)

Parameters:

- textPath – Input:path to the message file/where the message is

- glyphLib – Input: loaded library /where the letter library is

- scaleToMM – input: scale factor / the zoom factor

Return value: 1 if the entire file is processed and drawn per rules;0 on any error ( file open, bad glyph,or comms failure)

- int validateHeight(int textHeightMM)

Parameters:

- textHeightMM – input: proposed height

Return value: 1 if in [4,10]; 0 otherwise (caller prompts again or clamps).

Summary:

- Open the message file and call beginGcodeSession() to send the startup commands.

- Read the file one token at a time.

- If the token is CR(13), set cursorXmm = 0 ( go to the left edge)

- If the token is LF (10), call applyNewline(cursorXmm,cursorYmm,LINE_SPACING_MM)(move down one line).

- Otherwise, treat the token as a word:

- Get its width with estimateWordWidthMM(word,glyphLib, scaletoMM).

- If cursorXmm + width > MAX_DRAW_WIDTH_MM, start a new line before this word by calling applyNewline(…) and setting cursorXmm = 0.

- Then draw the word, character by character, with

- drawGlyph(c,glyphLib,scaleToMM, &cursorXmm,&cursorYmm, &penIsDown).

- At the end of the file, make sure the pen is up (S0) and send G0 X0 Y0 to return home.

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| loadGlyphLibrary | Valid font file loads/Font file exists and is well-formed. | fontPath = “SingleStrokeFont.txt”, empty glyphLib[128], libSize =128/Use the real font file and an empty 128-slot library | Return 1; spot-checks on entries looks sensible/ it loads without errors and a few sample letters look right |
| loadGlyphLibrary | File missing | fontPath= ”missing.txt”/pointing to a filename that doesn’t exist | Return 0, it fails gracefully |
| loadGlyphLibrary | Malformed header/ Font file starts with bad formatting. | First Block =! 999 CN or invalid N/ break the first line on purpose. | Return 0 / rejects the bad file. |
| computeScaleMM | Min height/ user choosing the smallest allowed size | textHeightMM = 4/ height =4mm | =0.222 (4/18.0) / gives the small zoom value |
| computeScaleMM | Max height/User choosing the largest allowed size | textHeight=10/ Height = 10mm | =0.5556 (10/18.0) / gives the larger zoom value. |
| computeScaleMM | Out of range/height not allowed | textHeightMM=3 or 11/ trying 3mm or 11mm | 0.0/Returns 0 so the caller can reject it. |
| estimateWordWidthMM | Known word width/Check if width maths is right | “HI”,advanceUnits(‘H’) = 18, advanceUnits(‘I’)=9,scaleToMM=0.5/ “HI” with made-up advances 18 and 9 at half scale. | 13.5mm = ((18+9) * 0.5) /Reports 13.5mm. |
| estimateWordWidthMM | Missing glyph in word/Word contains a character the font doesn’t have. | “HELLO#” (if # not in font)/ Adding a symbol the font lacks. | -1.0/ Signals can`t measure this word . |
| estimateWordWidthMM | Empty word/nothing to draw | “”/ Empty string | 0.0/Width is zero. |
| drawGlyph | Normal draw & advance/draw one letter and move the cursor forward. | C = “H”, valid glyphLib, scaleToMM =0.5, cursorXmm = 0, cursorYmm = 0, penIsDown =0/ start at (0,0), pen up. | Return 1;emits S1000 then G1…; cursorXmm += advanceUnits(‘H’)*0.5/Draws th H and shifts X by the letter’s width |
| drawGlyph | Glyph not found/the letter isn’t in the front. | C=’~’ and missing in glyphLib/Using a ltter the font doesn’t have | Return 0; no G-code sent/ Fails cleanly; doesn’t try to draw. |
| drawGlyph | No redundant pen toggles/Doesn’t spam the pen up or down when it isn’t needed | Glyph with consecutive pen=down strokes/ A letter that draws several lines without lifting | Return 1; no extra S1000/S0 between same-state strokes/only changes the pen when it actually needs to. |
| applyNewLine | Single newline/Go to the next line | cursorXmm=42, cursorYmm = -10 , lineSpacingMM = 5.0/ From X=42, Y=-10, move down one line. | cursorXmm = 0, cursorYmm =-15/ X resets to left edge, Y does down 5mm |
| applyNewLine | Blank line (double newline)/ Skip a line | Call twice/two newlines in a row. | cursorYmm decreases by 10.0mm total/moves down two line gaps. |
| beginGcodeSession | Startup accepted/Robot says “ok” to the setup | Device replies “ok\n” to each line/ simulating ok after each command. | Return 1; sends F1000, M3, S0 in order/ setup succeeded. |
| beginGcodeSession | Timeout/Robot doesn’t answer | No reply from device/pretend the robot is unplugged | Stop further sends/fails and stops. |
| emitGcode | One line success/Send a single move and get “ok” | Fmt =”G1 X%.3f Y%.3f\n”, 12.0, -5.0 / draw to (12.000, -5000). | Return 1; exactly G1 X12.000 Y-5.0000 sent; ”ok” received / command goes out and the robot confirms. |
| emitGcode | Serial error/sending fails. | Simulated write failure or timeout /pretending the cable is flaky | Return 0/ reports the failure. |
| processTextFile | Word fits on line/short word tha doesn’t need wrapping | File “HI”, MAX_DRAW_WIDTH_MM=100, LINE_SPACING_MM=5/plenty of room | Return 1; no wrap; finish s0 and G0 X0 Y0/draws it and ends pen-up at home. |
| processTextFile | Wrap before word/ Next word would go past 100mm so start a new line first | Next word width makes cursorXmm + width > 100/ put a long word at the edge | Return 1; calls applyNewLine(..) before drawing; cursorXmm=0/Jumps to a new line, then draws the word. |
| processTextFile | CR/LF handling/New line via CR then LF from the file | Bytes 13 then 10 between words/ Insert CR (13) then LF(10) | Return 1;CR ->cursorXmm= 0; LF -> cursorYmm -= 5.0/Left edge, then down one line |
| processTextFile | Unsupported character in file/ Message contains a letter not in the font | ~ is not in glyphLib/ include a missing symbol | Return 0/ stops processing |

# Flowchart(s)

All flowcharts are within the zip file.

# AI Statement

Tool used: ChatGPT

Dates used: 07–12 Nov 2025

Purpose: proofreading, terminology consistency, and logic sanity-checks. No code or design decisions were delegated. I wrote and verified all technical content.

How I used AI (high level)

- Proofreading only: grammar, spelling, and clarity rewrites for sentences I had already drafted.

- Consistency checks: flagged typos/mismatches in names (e.g., cursorXmm vs cusorXmm, applyNewline casing).

- Sanity-check of flow: asked to review the control-flow of my own flowcharts (e.g., where Yes/No branches return).

What I did not use AI for

- No code generation, no algorithm invention, no automated literature search, no auto-writing of data structures, prototypes, or tests.

- No external sources were inserted by AI. Any errors remain my responsibility.

Elements of the report where AI was used

- Outline wording: minor grammar fixes only.

- Key Data Items table: spelling/consistency of types and names (e.g., int16_t dy, LINE_SPACING_MM).

- Function Declarations: return-value wording clarity; caught typos in identifiers.

- Testing Information: added a plain-English line alongside the technical line.

- Flowcharts: checked that each diamond has a clear Yes/No path; suggested adding the “More strokes?” loop in drawGlyph.

Inputs I provided to the AI (examples)

Proofread request:

“Please proof-read the following outline paragraph for grammar only. Do not change technical meaning.”

(I pasted my own Outline text.)

Consistency check:

“Scan my Key Data Items table for typos in variable names and types. List exact fixes only.”

Flowchart sanity-check:

“In drawGlyph, after I compute targetX/targetY, confirm where Yes/No branches go and whether I loop for more strokes.”

Testing clarity:

“Rewrite the Expected Output for this row in plain English while keeping variable names and numeric values exactly.”

Representative outputs from the AI (examples)

- “Change cusorXmm → cursorXmm; ‘ApplyNewLine’ → applyNewline(...).”

- “Add a diamond ‘More strokes left?’ before the final advance; Yes loops back to the first pen-state check; No goes to cursorXmm += glyph.advanceUnits * scaleToMM then End.”

- “Expected Output (plain English): Returns 1; sends exactly G1 X12.000 Y-5.000; device replies ‘ok’; cursor updated accordingly.”

(No code was produced; outputs were short text suggestions like the above.)

My verification steps

- Re-read and accepted/rejected each suggestion manually.

- Cross-checked names against the tables and flowcharts.

- Ensured all logic still matched the MMME3085 brief (height/18 scaling, 100 mm wrap-before-word, CR/LF, F1000/M3/S0 startup, finish S0 + home).

I confirm that the design, data structures, function specifications, tests, and flowcharts are my own work and map directly to the module brief. AI was used only for proofreading, consistency notes, and non-technical wording. I take full responsibility for the content and any remaining errors.