# Rithik Subhash & 20486047

# Title

# Outline of the Problem to be Solved

The program will read a single-stroke font from `SingleStroke.txt’, which will be stored in memory, and then convert the input text into G-code that the drawing robot (via an Arduino) can execute. Each character in the font is defined by a sequence of relative moves (X, Y) with a pen state (0=up or 1 = down). At the run time, the user will supply a letter height in millimetres. The allowed range for that height is 4-10mm. A scale factor `height/18` is then applied to all font coordinates because the font’s nominal carp height is 18 units.

The program will read the message that will be written from a text file of arbitrary length. It will then process the file incrementally, so very long inputs are supported. The horizontal placement uses the final offset provided by each character’s data as in the last move set the start of the new character. Words will have to be laid out so they fit within a 100mm width drawing area without breaking words across the edge; if a new word exceeded 100mm, a new line is started before that word. Vertical layout uses a fixed line spacing of 5mm. The Carriage return (CR/ASCII 13) resets the X origin to the left margin; the line feed (LF/ASCII 10) moves the baseline down by 5mm. When dealing with the Y coordinate system, as the robot moves down the page, it makes Y more negative; going up the page makes Y more positive.

The G-code generation follows a small subset: The feedrate (F1000) and enable(M3) are sent once at the start. S0 raises the pen, S1000 lowers the pen. G0 X Y makes rapid non-drawing moves. G1 X Y draws lines. Pen commands are stateful and only sent when the state changes. Each G-code line is transmitted over a virtual serial port on Windows; the program essentially waits for an “okay” acknowledgement before sending the next line to respect the device buffer.

On completion, the program ensures the pen is up and returns the arm to (0,0). This submission is a design-only plan (no code): data structures, function prototypes, tests and flowcharts to guide the later implementation.

# Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |

Extend table as required

# Function Declarations

Only include functions that you will develop.

Example (remove before submission)

int TemperatureConversion( int InputTemp,  float* OutputTemp )

Parameters:

InputTemp – input temperature in degrees C

OutputTemp – pointer to return output temperature in degrees F

Return value – returns 1 if successful, 0 if failed

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |

Extend table as required. Note that ‘Function’ includes main()

# Flowchart(s)

May be included as separate pdf

# AI Statement