# Rithik Subhash & 20486047

# Title

# Outline of the Problem to be Solved

The program will read a single-stroke font from `SingleStroke.txt’, which will be stored in memory, and then convert the input text into G-code that the drawing robot (via an Arduino) can execute. Each character in the font is defined by a sequence of relative moves (X, Y) with a pen state (0=up or 1 = down). At the run time, the user will supply a letter height in millimetres. The allowed range for that height is 4-10mm. A scale factor `height/18` is then applied to all font coordinates because the font’s nominal carp height is 18 units.

The program will read the message that will be written from a text file of arbitrary length. It will then process the file incrementally, so very long inputs are supported. The horizontal placement uses the final offset provided by each character’s data as in the last move set the start of the new character. Words will have to be laid out so they fit within a 100mm width drawing area without breaking words across the edge; if a new word exceeded 100mm, a new line is started before that word. Vertical layout uses a fixed line spacing of 5mm. The Carriage return (CR/ASCII 13) resets the X origin to the left margin; the line feed (LF/ASCII 10) moves the baseline down by 5mm. When dealing with the Y coordinate system, as the robot moves down the page, it makes Y more negative; going up the page makes Y more positive.

The G-code generation follows a small subset: The feedrate (F1000) and enable(M3) are sent once at the start. S0 raises the pen, S1000 lowers the pen. G0 X Y makes rapid non-drawing moves. G1 X Y draws lines. Pen commands are stateful and only sent when the state changes. Each G-code line is transmitted over a virtual serial port on Windows; the program essentially waits for an “okay” acknowledgement before sending the next line to respect the device buffer.

On completion, the program ensures the pen is up and returns the arm to (0,0). This submission is a design-only plan (no code): data structures, function prototypes, tests and flowcharts to guide the later implementation.

# Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| Stroke | struct {int16_t dx; int16_dy; uint8_t pen;} | Encodes one relative move from the font (change in x, change in y) plus pen state (0/1). Fixed-width ints match small coordinate ranges and avoid waste. One step in drawing a letter: how far to move in X and Y, and whether the pen is up or down. |
| Glyph | struct { uint8_t ascii; uint16_t nStrokes; Stroke* strokes; int16_t advanceUnits; } | Holds all strokes for one ASCII character; dynamic array because stroke count varies. advanceUnits caches the character’s horizontal advance (usually last X) for quick width estimates. Everything needed to draw one letter, plus how much space it takes before the next letter starts. |
| glyphLib | Glyph[128] | Direct ASCII-indexed lookup table for all glyphs (0(1) access during generation). A ready to use shelf of every letter so we can grab any letter`s recipe fast. |
| textHeightMM | int | User input(4-10mm). Integer keeps validation simple and matches spec. The size of the letters in millimetres that the user chooses. |
| scaletoMM | double | textHeightMM/18.0 converts font units to millimetres with sub-mm precision. The “zoom level” that turns font numbers into real sizes on paper. |
| cursorXmm, cursorYmm | double | The absolute drawing origin for the next stroke/glyph; required to convert relative font moves into absolute G-code coordinates. Tells where the pen is right now on the page (left-right and up-down in mm). |
| penIsDown | uint8_t | Tracks the pen state to suppress redundant S0/S1000 emissions. Shows whether the pen is currently touching the paper. |
| LINE_SPACING_MM | const double | Fixed inter-line offset to 5mm per spec; avoids magic numbers and centralises a constraint. The gap between lines of text. |
| MAX_DRAW_WIDTH_MM | const double | 100 is the hard limit for line wrapping. Used to decide wrap before drawing. The maximum width allowed for a line; if a word would exceed it, a new line would be started. |
| ASCII_CR, ASCII_LF | const int | The explicit control codes to reset X 13 (CR) and step Y by line spacing 10 (LF). Special new line signals in the text file. |
| gcLine | char[100] | Reusable formatting buffer for ASCII G-code lines before sending. A small text box where we build each command to the robot. |
| COM_INDEX | int | RS-232 library handle/index for the Windows virtual COM port (open/write/read). Which USB/COM port the robot is plugged into. |
| wordWidthMM | double | Temporary accumulator that sums advanceUnits * scaleToMM for the next word to decide wrap. A quick calculator to check if the next word will still fit on this line. |
| fontPath,textPath | char[] | File locations for the font and input text; enables testing with different sources. Where the font file and the message file live on your computer. |

# Function Declarations

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |

Extend table as required. Note that ‘Function’ includes main()

# Flowchart(s)

May be included as separate pdf

# AI Statement